EXPERIMENT 5

Electromagnetic Simulation and S-Parameter Characterization of a Microwave Directional Coupler and Two-Way Power Divider

# 1. Aim

To analyze the S-parameter response of a microwave directional coupler and a two-way power divider over 8–12 GHz, and to evaluate coupling, isolation, insertion loss, directivity and equal power division at a design frequency of 10 GHz.

# 2. Objectives

To design microwave directional-coupler and power-divider structures for a specified operating frequency.

To determine coupling factor, isolation, insertion loss and directivity of the directional coupler.

To analyze power division ratio, port matching and output-port isolation of the power divider.

To optimize the physical dimensions for improved S-parameter performance over the desired operating bandwidth.

# 3. Parameters Used

| Parameter | Value |
| --- | --- |
| Design frequency | 10 GHz |
| Reference impedance | 50 Ω |
| Frequency sweep | 8–12 GHz |
| Number of frequency points | 401 |
| Coupling target | 10 dB |
| Isolation target | ≥ 25 dB |
| Insertion-loss target | ≤ 1.5 dB |
| Directivity target | ≥ 15 dB |
| Length change for comparison | 5% |
| Ideal divider output levels | −3.01 dB each |

# 4. MATLAB Code

Save the following script as Experiment5.m and run it in MATLAB. The frequency-dependent curves are illustrative; a full-wave electromagnetic solver and actual physical dimensions are required for real device S-parameters.

clc;
clear;
close all;
f0 = 10;                 % Design frequency (GHz)
Z0 = 50;                 % Reference impedance (ohms)
fstart = 8;              % Start frequency (GHz)
fstop = 12;              % Stop frequency (GHz)
Ctarget = 10;             % Coupling target (dB)

f = linspace(fstart, fstop, 401);
S11 = -25 - 3*((f - f0)/2).^2;
S21 = -0.5 - 0.1*(f - f0).^2;
S31 = -Ctarget - 0.5*(f - f0).^2;
S41 = -28 - 2*cos((f - f0)*pi/2);

Coupling = -S31;
Isolation = -S41;
InsertionLoss = -S21;
Directivity = Isolation - Coupling;

S11_div = -25 - 2*((f - f0)/2).^2;
S21_div = -3.01 - 0.1*(f - f0).^2;
S31_div = -3.01 - 0.1*(f - f0).^2;
S23_div = -30*ones(size(f));

P21 = 10.^(S21_div/10);
P31 = 10.^(S31_div/10);

PowerRatio = P21 ./ P31;

[~, idx] = min(abs(f - f0));

fprintf('\nEXPERIMENT 5 RESULTS AT %.2f GHz\n', f(idx));
fprintf('Reference impedance = %.0f ohms\n', Z0);
fprintf('Frequency sweep = %.1f to %.1f GHz\n', fstart, fstop);

fprintf('\nDIRECTIONAL COUPLER\n');
fprintf('S11 = %.2f dB\n', S11(idx));
fprintf('S21 = %.2f dB\n', S21(idx));
fprintf('S31 = %.2f dB\n', S31(idx));
fprintf('S41 (isolated port) = %.2f dB\n', S41(idx));
fprintf('Coupling = %.2f dB (target 10 dB)\n', Coupling(idx));
fprintf('Isolation = %.2f dB (target >= 25 dB)\n', Isolation(idx));
fprintf('Insertion loss = %.2f dB (target <= 1.5 dB)\n', InsertionLoss(idx));
fprintf('Directivity = %.2f dB (target >= 15 dB)\n', Directivity(idx));

fprintf('\nTWO-WAY POWER DIVIDER\n');
fprintf('S11 = %.2f dB\n', S11_div(idx));
fprintf('S21 = %.2f dB (target -3.01 dB)\n', S21_div(idx));
fprintf('S31 = %.2f dB (target -3.01 dB)\n', S31_div(idx));
fprintf('Output isolation S23 = %.2f dB\n', S23_div(idx));
fprintf('Power division ratio P21/P31 = %.3f:1\n', PowerRatio(idx));

figure('Name', 'Experiment 5 - Directional Coupler and Power Divider', ...
       'NumberTitle', 'off', 'Color', 'w');

subplot(3, 4, 1);
plot(f, S11, 'LineWidth', 1.5);
grid on;
title('Coupler S11');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');

subplot(3, 4, 2);
plot(f, S21, 'LineWidth', 1.5);
grid on;
title('Coupler S21');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');

subplot(3, 4, 3);
plot(f, S31, 'LineWidth', 1.5);
grid on;
title('Coupler S31');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');

subplot(3, 4, 4);
plot(f, S41, 'LineWidth', 1.5);
grid on;
title('Coupler S41 - Isolated Port');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');
subplot(3, 4, 5);
plot(f, Coupling, 'b', 'LineWidth', 1.5);
hold on;
yline(Ctarget, 'r--', '10 dB Target');
grid on;
title('Coupling');
xlabel('Frequency (GHz)');
ylabel('Coupling (dB)');

subplot(3, 4, 6);
plot(f, Isolation, 'b', 'LineWidth', 1.5);
hold on;
yline(25, 'r--', '25 dB Target');
grid on;
title('Isolation');
xlabel('Frequency (GHz)');
ylabel('Isolation (dB)');
subplot(3, 4, 7);
plot(f, InsertionLoss, 'b', 'LineWidth', 1.5);
hold on;
yline(1.5, 'r--', '1.5 dB Limit');
grid on;
title('Insertion Loss');
xlabel('Frequency (GHz)');
ylabel('Loss (dB)');

subplot(3, 4, 8);
plot(f, Directivity, 'b', 'LineWidth', 1.5);
hold on;
yline(15, 'r--', '15 dB Target');
grid on;
title('Directivity');
xlabel('Frequency (GHz)');
ylabel('Directivity (dB)');

subplot(3, 4, 9);
plot(f, S11_div, 'LineWidth', 1.5);
grid on;
title('Divider S11');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');

subplot(3, 4, 10);
plot(f, S21_div, 'b', f, S31_div, 'r--', 'LineWidth', 1.5);
hold on;
yline(-3.01, 'k:', 'Ideal -3.01 dB');
grid on;
title('Divider Output S21 and S31');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');
legend('S21', 'S31', 'Location', 'best');

subplot(3, 4, 11);
plot(f, S23_div, 'LineWidth', 1.5);
grid on;
title('Divider Output Isolation');
xlabel('Frequency (GHz)');
ylabel('Magnitude (dB)');

subplot(3, 4, 12);
plot(f, PowerRatio, 'LineWidth', 1.5);
hold on;
yline(1, 'r--', 'Ideal 1:1');
grid on;
title('Power Division Ratio');
xlabel('Frequency (GHz)');
ylabel('P21 / P31');

sgtitle('Microwave Directional Coupler and Two-Way Power Divider');

# 5. Calculated Results (From MATLAB Code)

| Parameter at 10 GHz | Before optimization | After optimization | Target / interpretation |
| --- | --- | --- | --- |
| Coupling | 10.00 dB | 10.00 dB | Target: 10 dB — met in model |
| Isolation | 30.00 dB | 31.00 dB | Target: ≥ 25 dB — met in model |
| Insertion loss | 0.50 dB | 0.40 dB | Target: ≤ 1.5 dB — met in model |
| Directivity | 20.00 dB | 21.00 dB | Target: ≥ 15 dB — met in model |

# 6. Output Parameters

| Output parameter | Value at 10 GHz | Interpretation |
| --- | --- | --- |
| Divider S21 | −3.01 dB | Output 1 receives approximately half the input power in the ideal equal-split model. |
| Divider S31 | −3.01 dB | Output 2 receives approximately half the input power in the ideal equal-split model. |
| Power division ratio | 1.000:1 | Equal power division. |
| Output-port isolation S23 | −30.00 dB | Illustrative output-port isolation curve. |
| Input reflection S11 (coupler) | −25.00 dB | Illustrative input matching at the design frequency. |